Sharp Precision Machine Tools

Proposal

## PRECISION LATHE

2660C

Sharp 2660C Lathe:					$ 31,200.00

STANDARD EQUIPMENT:

- 4-Jaw Chuck 14”

- Coolant System

- 4 way toolpost

- Halogen Work Light

- Face Plate 20”

- Steady Rest (3/4” to 9”)

- 4-Way Carriage Stop

- Splash Guard

- Chuck Guard

- Tool Kit

FEATURES:

- D1-8 Camlock

- 3-1/8” Spindle Bore

- Direct Selection For Inch/Metric Threads

- One Piece Cast Meehanite Bed

- 15 Hp Spindle Motor

- Single Speed to Start Motor

- 12-Speed Gear Head (15-1,500 Rpm)

SPECIFICATIONS:

- Capacity

- Swing Over Bed								26”

- Swing Over Cross Slide							17.24”

- Center Height                                                                                                           13”

- Swing Over Gap, width							35”

- Distance Between Centers							60”

- Headstock

- Spindle Nose Type								D1-8 camlock

- Spindle Bore									3-1/8”

- Spindle Speeds (12)								15-1,500 rpm

- Taper of Spindle Bore								MT#7

- Taper of Center								MT#5

- Threads and Feeds

- Longitudinal Feeds – in/rev (18)						0.002 - 0.032”

- Cross Feeds – in / rev {#}							0.005 –0.018” {17}

- Inch Threads (#)								2-56 tpi {44}

- Metric and Module Threads {#} 						0.5-14mm {24}

- D.P. Threads (36)								4-56 mm

- Leadscrew Pitch, Diameter							4 tpi, 1-1/2”

- Carriage

- Cross Slide Travel								14.75”

- Compound Rest Travel							8”

- Compound Width								6”

- Bed

- Bed Length									110.5”

- Bed Width									15.75”

- Tailstock

- Quill Travel (graduated)							6-1/2”

- Quill Diameter									3.34”

- Quill Taper									MT#5

- General

- Spindle Motor (220V or 440V, 60, 3)						15 hp

- Net Weight (approx.)								5830 Lbs

- Dimensions ( L x W x H)							114”x40.5”x51.5”

- Standard Color									Gray

- Specifications may be changed or improved without notice.

- Total Power Consumptions

- 230V 							                      14KVA/40 amps

- 460V							                      14KVA/20 amps

OPTIONAL ACCESSORIES:

- 3 - Jaw Chuck 10” (Bison)					803-1038		$ 1,545.00

- 3 - Jaw Chuck 12” (Bison)					803-1238		$ 2,380.00

- 3 - Jaw Chuck 16” (Bison)					803-1638		$ 4,280.00

- 4 - Jaw Chuck 16” (Bison)					853-1638		$ 1,835.00

- Follow Rest						99120			$    315.00

- Roller jaws (set of 3)					26C-RJAW		$    630.00

- Quick Change Tool Holder Set (Aloris, CA#4)		CA-2511-7350		$ 2,020.00

- Taper Attachment (Factory Installed)			991126			$ 1,945.00

- Drill & Arbor (JACOBS)					20/26C-D/A		$  675.00

- Live Center – Royal (Dual Bearing) MT#5		10855			$   280.00

- Live Center – MT#5					LC-MT5		$   150.00

- 5C COLLET CLOSER					CC-2680C		$ 1,090.00

Installation – 5c Collet Closer							$     75.00 NET

- 5C collet set (1/8”-1-1/16” by 1/16”), 16 pcs		ACC-5C		$   295.00

- Steady Rest 15”						22C-STEADY15	$ 1,960.00

- Steady Rest 19.5” for 26” Lathe Only			22c-steady20		$ 2,610.00

- Micrometer carriage stop					20C-02			$   250.00

# DIGITAL READOUT SYSTEMS:

- Fagor 20i D.R.O. (installed)*				FG-2680		$ 2,765.00

- Acu-Rite 200S D.R.O. 					AR-2680		$ 3,279.00

- Newall DP700 D.R.O.					NWDP-2680		$ 3,565.00

Factory Installation - D.R.O.								$    525.00 NET

* Fagor DRO price includes installation

Note:  All prices and specification subject to change without notice.  Please verify